http://www.yourcobalt.com/forums/problems-service/19543-torque-specs-fluid-capacities.html

Approximate Fluid Capacities

The following approximate capacities are given in English and metric conversions. See Fluid and Lubricant Recommendations for more information. All capacities are approximate. When adding, be sure to fill to the approximate level, as recommended in this manual. Recheck fluid level after filling.

Cooling System

•2.0L L4 Engine Supercharged - RPO LSJ

5.7 liters

6.0 quarts

•2.2L L4 Engine - RPO L61

6.5 liters

6.8 quarts

•2.4L L4 Engine - RPO LE5

7.0 liters

7.4 quarts

Intercooler System 2.0L Engine - RPO LSJ

1.9 liters

2.0 quarts

Engine Oil With Filter

•2.2L L4 Engine - RPO L61

4.8 liters

5.0 quarts

•2.0L L4 Engine Supercharged - RPO LSJ

6.0 liters

6.3 quarts

•2.4L L4 Engine - RPO LE5

4.8 liters

5.0 quarts

Fuel Tank

51.1 liters

13.5 gallons

Transmission/Transaxle

•Transaxle, Automatic- Bottom Pan Removed 4T40-E

6.6 liters

7.0 quarts

•Dry

12.2 liters

12.9 quarts

•Complete Overhaul

9.0 liters

9.5 quarts

•Transaxle, Manual - Getrag 5 Speed

1.7 liters

1.8 quarts

•Transaxle, Manual - MU3

1.9 liters

2.0 quarts

Fastener Tightening Specifications

steering

power steering

Intermediate Steering Shaft Bolt

34 N·m

25 lb ft

Power Steering Assist Motor Bolt

19 N·m

14 lb ft

Rack and Pinion Outer Tie Rod End Nut

25 N·m + 90 degrees

18 lb ft + 90 degrees

Steering Gear Bolt

110 N·m

81 lb ft

steering wheel and column

Intermediate Steering Shaft Bolt

34 N·m

25 lb ft

Steering Column Pivot Bolt

25 N·m

18 lb ft

Steering Column Mounting Bolt

25 N·m

18 lb ft

Steering Column Trim Cover Screw

2 N·m

18 lb in

Steering Column Jacket Bolt

11 N·m

97 lb in

Steering Wheel Nut

41 N·m

30 lb ft

front suspension

Ball Joint to Control Arm Nut and Bolts

70 N·m

52 lb ft

Ball Joint to Steering Knuckle Nut

•First Pass

50 N·m

Reverse Nut 3/4 turn

37 lb ft

Reverse Nut 3/4 turn

•Second Pass

50 N·m + 30°

37 lb ft + 30°

Control Arm to Front Frame Bolts

55 N·m

41 lb ft

Control Arm to Rear Frame Bolts

100 N·m + 180°

74 lb ft + 180°

Frame to Chassis Bolts

100 N·m + 180°

74 lb ft + 180°

Intermediate Shaft Pinch Bolt

34 N·m

25 lb ft

Outer Tie Rod End Nut

25 N·m + 90°

18 lb ft + 90°

Stabilizer Link to Strut Nut

65 N·m

48 lb ft

Stabilizer Shaft Clamp Bolt

50 N·m

37 lb ft

Stabilizer Shaft Link to Stabilizer Bar Nut

80 N·m

59 lb ft

Strut Assembly to Steering Knuckle Nut

120 N·m

89 lb ft

Strut to Body Nut

20 N·m

15 lb ft

Strut Shaft Nut

70 N·m

52 lb ft

Transmission Mount Through Bolt

100 N·m

74 lb ft

Transmission Mount to Frame Bolt

50 N·m

37 lb ft

Wheel Bearing/Hub Assembly Mounting Bolts

115 N·m

85 lb ft

rear suspension

Bracket to Body Bolts

90 N·m + 45°

66 lb ft + 45°

Brake Hose Fittings

19 N·m

14 lb ft

Bushing Through Bolts

90 N·m + 60°

66 lb ft + 60°

Shock Bolt Lower

110 N·m

81 lb ft

Shock Bolt Upper

90 N·m

66 lb ft

Wheel Bearing/Hub Assembly Mounting Nuts

45 N·m + 30°

33 lb ft + 30°

wheels

Wheel Nuts

140 N·m

100 lb ft

axles

Intermediate Drive Shaft Bracket-to-Engine Block Bolts

50 N·m

37 lb ft

Wheel Drive Shaft Nut

210 N·m

155 lb ft

brakes

Accelerator Pedal and Sensor Assembly Mounting Bolts

9 N·m

80 lb in

Brake Hose-to-Brake Caliper Bolt

48 N·m

35 lb ft

Brake Master Cylinder Mounting Nuts

25 N·m

18 lb ft

Brake Pedal Bracket-to-Steering Column Bracket Bolt

25 N·m

18 lb ft

Brake Pipe Fitting at the Front Brake Hose

19 N·m

14 lb ft

Brake Pipe Fittings at the Master Cylinder

19 N·m

14 lb ft

Brake Pipe Fittings at the Proportioning Valve Assembly

19 N·m

14 lb ft

Brake Pipe Fittings at the Rear Brake Hose

19 N·m

14 lb ft

Front Brake Hose Bracket Retaining Bolt

25 N·m

18 lb ft

Proportioning Valve Assembly Bracket Mounting Bolt

25 N·m

18 lb ft

Vacuum Brake Booster Mounting Nuts

17 N·m

14 lb ft

Brake Caliper Bleeder Valve

11 N·m

97 lb in

Brake Caliper Guide Pin Bolt

34 N·m

25 lb ft

Brake Caliper Mounting Bracket Bolt

115 N·m

85 lb ft

Brake Hose-to-Brake Caliper Bolt

48 N·m

35 lb ft

Brake Pipe Fitting at the Wheel Cylinder

19 N·m

14 lb ft

Wheel Cylinder Bleeder Valve

8 N·m

71 lb in

Wheel Cylinder Mounting Bolts

16 N·m

12 lb ft

Park Brake Cable Adjusting Nut, Drum Brakes

3.9 N·m

35 lb in

Park Brake Lever Assembly Mounting Nuts, Tighten in Sequence

25 N·m

18 lb ft

Park Brake Warning Lamp Switch Mounting Screw

3 N·m

27 lb in

Rear Park Brake Cable Guide Retaining Bolt

10 N·m

89 lb in

Brake Pipe Fittings at the Brake Pressure Modulator Valve (BPMV)

19 N·m

14 lb ft

Brake Pressure Modulator Valve Bracket Mounting Bolts

25 N·m

18 lb ft

Brake Pressure Modulator Valve-to-Bracket Mounting Bolt

5.6 N·m

50 lb in

Electronic Brake Control Module (EBCM) to Brake Pressure Modulator Valve (BPMV) Retaining Bolts

4.5 N·m

40 lb in

Engine Control Module Bracket Mounting Bolts

12 N·m

106 lb in

engine 2.0l lsj
A/C Compressor to Block Bolt
20 N·m
15 lb ft

Accessory Drive Belt Tensioner Bolt
32 N·m
24 lb ft

Balance Shaft Adjustable Chain Guide Bolt
15 N·m
11 lb ft

Balance Shaft Bearing Carrier to Block Bolt
10 N·m
89 lb in

Balance Shaft Fixed Chain Guide Bolt
15 N·m
11 lb ft

Balance Shaft Sprocket Bolt
50 N·m
37 lb ft

Baro Sensor and Coolant Return Line Assembly
8 N·m
71 lb in

Boost Solenoid Bracket Nut
10 N·m
89 lb in

Cam Cover to Cylinder Head Bolt
8 N·m
71 lb in

Cam Cover to Ground Cable Bolt
10 N·m
89 lb in

Cam Cover to Ground Cable Stud
10 N·m
89 lb in

Cam Sensor Bolt
8 N·m
71 lb in

Camshaft Position Sensor Housing Stud
22 N·m
16 lb ft

Camshaft Bearing Cap Bolt
10 N·m
89 lb in

Camshaft Sprocket Bolt

•First Pass
85 N·m
63 lb ft

•Final Pass
30 degrees

Camshaft Timing Chain Tensioner
75 N·m
55 lb ft

Chain Guide Access Hole Plug
90 N·m
59 lb ft

Connecting Rod Bolts

•First Pass
25 N·m
18 lb ft

•Final Pass
100 degrees

Crankshaft Bearing Lower Crankcase to Block Bolts

•First Pass
20 N·m
15 lb ft

•Final Pass
70 degrees

Crankshaft Position Sensor Bolt
8 N·m
71 lb in

Crankshaft Position Sensor Housing Studs
22 N·m
16 lb ft

Crankshaft Pulley Bolt

•First Pass
100 N·m
74 lb ft

•Final Pass
75 degrees

Cylinder Head Bolt

•First Pass
30 N·m
22 lb ft

•Final Pass
155 degrees

Cylinder Head Front Chaincase Bolt
35 N·m
26 lb ft

Cylinder Head Oil Gallery Plug
35 N·m
26 lb ft

Dipstick Guide to Intake Manifold Bolt
10 N·m
89 lb in

Elek. ICM Cover Bolt
8 N·m
71 lb in

Engine Coolant Temperature Sensor
22 N·m
16 lb ft

Engine Lift Bracket Front Bolt
25 N·m
18 lb ft

Engine Lift Bracket Rear Bolt
25 N·m
18 lb ft

Engine Mount Intermediate Bracket Bolts
100 N·m
74 lb ft

Engine Mount to Intermediate Bracket Bolts
50 N·m
37 lb ft

Engine Mount to Mid-Rail Nuts
100 N·m
74 lb ft

Exhaust Manifold to Cylinder Head Nut
12 N·m
106 lb in

Exhaust Manifold to Cylinder Head Stud
10 N·m
89 lb in

Exhaust Manifold Pipe Flange Stud
16 N·m
12 lb ft

Exhaust Takedown Pipe Nuts
30 N·m
22 lb ft

Flywheel (SMT) Bolt

•First Pass
53 N·m
39 lb ft

•Final Pass
25 degrees

Frame Bolts

•First Pass
100 N·m
74 lb ft

•Second Pass
180 degrees

Front Cover to Block Bolt
25 N·m
18 lb ft

Front Lift Bracket Bolt
25 N·m
18 lb ft

Fuel Feed Line
14 N·m
124 lb in

Fuel Feed Line and Injector Harness Bracket
10 N·m
89 lb in

Fuel Line Bracket Bolt
10 N·m
89 lb in

Fuel Pipe Bracket Bolt
10 N·m
89 lb in

Fuel Rail Bracket Stud
10 N·m
89 lb in

Generator Bracket Bolt
42 N·m
31 lb ft

Generator Connector Nut
20 N·m
15 lb ft

Generator to Block Bolt
20 N·m
15 lb ft

Heat Shield to Exhaust Manifold Bolt
23 N·m
17 lb ft

Idler Pulley
22 N·m
16 lb ft

Ignition Coil Bolt
8 N·m
71 lb in

Intake Camshaft Rear Cap Bolt
25 N·m
18 lb ft

Intake Manifold to Cylinder Head Bolt
25 N·m
18 lb ft

Intake Manifold to Cylinder Head Nut
25 N·m
18 lb ft

Intake Manifold to Cylinder Head Stud
6 N·m
53 lb in

Knock Sensor Bolt
25 N·m
18 lb ft

Lower Crankcase to Block Peripheral Bolts
25 N·m
18 lb ft

Oil Bypass Tube
22 N·m
16 lb ft

Oil Cooler
22 N·m
16 lb ft

Oil Filter Housing Cover
25 N·m
18 lb ft

Oil Gallery Plug
35 N·m
26 lb ft

Oil Gallery Plug - Rear
60 N·m
44 lb ft

Oil Level Indicator Tube Bolt
10 N·m
89 lb in

Oil Pan Drain Plug
25 N·m
18 lb ft

Oil Pan to Block Bolts
25 N·m
18 lb ft

Oil Pressure Sensor
18 N·m
13 lb ft

Oil Pump Gerotor Cover Bolt
6 N·m
53 lb in

Oil Pump Pressure Relief Valve Plug
40 N·m
30 lb ft

Oxygen Sensor
42 N·m
31 lb ft

Power Steering Pump Blockout Plate Bolts
25 N·m
18 lb ft

Rear Lift Bracket Bolt
22 N·m
16 lb ft

Spark Plug
20 N·m
15 lb ft

Starter Motor to Block Bolt
40 N·m
30 lb ft

Starter Terminal Nut
17 N·m
13 lb ft

Supercharger Bolts
25 N·m
18 lb ft

Thermostat Housing to Block Bolts
10 N·m
89 lb in

Throttle Body Bolt
10 N·m
89 lb in

Timing Chain Adjustable Guide Bolt
10 N·m
89 lb in

Timing Chain Fixed Guide Bolt
15 N·m
11 lb ft

Timing Chain Oil Nozzle Bolt
10 N·m
89 lb in

Timing Chain Upper Guide Bolt
10 N·m
89 lb in

Torque Converter Bolts
62 N·m
46 lb ft

Transmission to Engine Bolts
75 N·m
55 lb ft

Vent Tube to Cylinder Head
15 N·m
11 lb ft

Water Pipe Support Bracket Bolt
10 N·m
89 lb in

Water Pump Access Cover Bolt
7 N·m
62 lb in

Water Pump/Balance Shaft Chain Tensioner Bolt
10 N·m
89 lb in

Water Pump Bolts
25 N·m
18 lb ft

Water Pump Drain Plug
20 N·m
15 lb ft

Water Pump Sprocket Bolt
10 N·m
89 lb in

engine 2.2l l61
A/C Compressor to Block Bolt
20 N·m
15 lb ft

Balance Shaft Adjustable Chain Guide Bolt
15 N·m
11 lb ft

Balance Shaft Bearing Carrier to Block Bolt
10 N·m
89 lb in

Balance Shaft Fixed Chain Guide Bolt
15 N·m
11 lb ft

Balance Shaft Sprocket Bolt
50 N·m
37 lb ft

Block Heater Bolt
10 N·m
89 lb in

Cam Cover to Cylinder Head Bolt
10 N·m
89 lb in

Cam Cover to Ground Cable Bolt
10 N·m
89 lb in

Cam Cover to Ground Cable Stud
10 N·m
89 lb in

Camshaft Cap Bolt
10 N·m
89 lb in

Camshaft Sprocket Bolt

•First Pass
85 N·m
63 lb ft

•Final Pass
30 degrees

Camshaft Timing Chain Tensioner
75 N·m
55 lb ft

Chain Guide Plug
90 N·m
59 lb ft

Connecting Rod Bolt

•First Pass
25 N·m
18 lb ft

•Final Pass
100 degrees

Crankshaft Bearings - Lower Crankcase to Block - Bedplate

•First Pass
20 N·m
15 lb ft

•Final Pass
70 degrees

Crankshaft Balancer Bolt

•First Pass
100 N·m
74 lb ft

•Final Pass
125 degrees

Crankshaft Position Sensor Bolt
10 N·m
89 lb in

Cylinder Head Bolt

•First Pass
30 N·m
22 lb ft

•Final Pass
155 degrees

Cylinder Head Front Chaincase Bolt
35 N·m
26 lb ft

Cylinder Head Oil Gallery Plug
35 N·m
26 lb ft

Drive Belt Tensioner Bolt
45 N·m
33 lb ft

EGR Cover Bolt
25 N·m
18 lb ft

Engine Coolant Temperature Sensor
22 N·m
16 lb ft

Engine Lift Bracket Front Bolt
25 N·m
18 lb ft

Engine Lift Bracket Rear Bolt
25 N·m
18 lb ft

EVAP Purge Solenoid Valve Nut
18 N·m
13 lb ft

Exhaust Manifold to Cylinder Head Nut
14 N·m
124 lb in

Exhaust Manifold to Cylinder Head Stud
10 N·m
89 lb in

Exhaust Manifold Pipe Flange Stud
16 N·m
12 lb ft

Flexplate (AMT) Bolt

•First Pass
53 N·m
39 lb ft

•Final Pass
25 degrees

Flywheel (SMT) Bolt

•First Pass
53 N·m
39 lb ft

•Final Pass
25 degrees

Front Cover to Block Bolt
25 N·m
18 lb ft

Front Lift Bracket Bolt
25 N·m
18 lb ft

Fuel Pipe Bracket Bolt
10 N·m
89 lb in

Fuel Rail Bracket Bolt
10 N·m
89 lb in

Generator to Block Bolt
23 N·m
17 lb ft

Heat Shield to Exhaust Manifold Bolt
23 N·m
17 lb ft

Ignition Coil Bolt
10 N·m
89 lb in

Intake Camshaft Rear Cap Bolt
25 N·m
18 lb ft

Intake Manifold to Cylinder Head Bolt
10 N·m
89 lb in

Intake Manifold to Cylinder Head Nut
10 N·m
89 lb in

Intake Manifold to Cylinder Head Stud
6 N·m
53 lb in

Knock Sensor Bolt
25 N·m
18 lb ft

Lower Crankcase to Block Perimeter Bolt
25 N·m
18 lb ft

Oil Filter Housing Cover
25 N·m
18 lb ft

Oil Gallery Gerotor Cover - Rear Bolt
6 N·m
53 lb in

Oil Gallery Plug
35 N·m
26 lb ft

Oil Gallery Plug - Rear
60 N·m
44 lb ft

Oil Level Indicator Tube to Intake Manifold Bolt
10 N·m
89 lb in

Oil Pan Drain Plug
25 N·m
18 lb ft

Oil Pan to Block Bolts
25 N·m
18 lb ft

Oil Pressure Switch
22 N·m
16 lb ft

Oil Pump Cover Bolt
6 N·m
53 lb in

Oil Pump Pressure Relief Valve Plug
40 N·m
30 lb ft

Oxygen Sensor
42 N·m
31 lb ft

Power Steering Pump Bolt
25 N·m
18 lb ft

Spark Plug
20 N·m
15 lb ft

Starter Motor to Block Bolt
53 N·m
39 lb ft

Thermostat Housing to Block Bolts
10 N·m
89 lb in

Throttle Body Bolt
10 N·m
89 lb in

Throttle Body Nut
10 N·m
89 lb in

Throttle Body Stud
6 N·m
53 lb in

Timing Adjustable Chain Guide Bolt
10 N·m
89 lb in

Timing Chain Oil Nozzle Bolt
10 N·m
89 lb in

Timing Fixed Chain Guide Bolt
15 N·m
11 lb ft

Timing Upper Chain Guide Bolt
10 N·m
89 lb in

Vent Tube to Cylinder Head
15 N·m
11 lb ft

Water Jacket Drain Plug
20 N·m
15 lb ft

Water Pipe Support Bracket Bolt
10 N·m
89 lb in

Water Pump Access Cover Bolt
7 N·m
62 lb in

Water Pump/Balance Shaft Chain Tensioner Bolt
10 N·m
89 lb in

Water Pump Bolts
25 N·m
18 lb ft

Water Pump Sprocket Bolt
10 N·m
89 lb in

2.4l le5
A/C Compressor to Block Bolt
20 N·m
15 lb ft

Balance Shaft Adjustable Chain Guide Bolt
15 N·m
11 lb ft

Balance Shaft Bearing Carrier to Block Bolt
10 N·m
89 lb in

Balance Shaft Fixed Chain Guide Bolt
15 N·m
11 lb ft

Balance Shaft Sprocket Bolt
50 N·m
37 lb ft

Block Core Plug
40 N·m
30 lb ft

Block Heater Bolt
10 N·m
89 lb in

Cam Cover to Cylinder Head Bolt
10 N·m
89 lb in

Cam Cover to Ground Cable Bolt
10 N·m
89 lb in

Cam Cover to Ground Cable Stud
10 N·m
89 lb in

Camshaft

•Camshaft Cap Bolt
10 N·m
89 lb in

•Camshaft Position Actuator Solenoid Valve Bolt
10 N·m
89 lb in

•Camshaft Position Sensor Bolt
10 N·m
89 lb in

•Camshaft Timing Chain Tensioner
75 N·m
55 lb ft

•Exhaust Camshaft Position Actuator -- First Pass
30 N·m
22 lb ft

•Exhaust Camshaft Position Actuator -- Final Pass
100 degrees

•Intake Camshaft Position Actuator -- First Pass
30 N·m
22 lb ft

•Intake Camshaft Position Actuator -- Final Pass
100 degrees

•Intake Camshaft Rear Cap Bolt
25 N·m
18 lb ft

Chain Guide Plug
90 N·m
59 lb ft

Connecting Rod Bolt

•First Pass
25 N·m
18 lb ft

•Final Pass
100 degrees

Crankshaft Balancer Bolt

•First Pass
100 N·m
74 lb ft

•Final Pass
125 degrees

Crankshaft Bearings - Lower Crankcase to Block - Bedplate

•First Pass
20 N·m
15 lb ft

•Final Pass
70 degrees

Crankshaft Position Reluctor Ring
15 N·m
11 lb ft

Crankshaft Position Sensor Bolt
10 N·m
89 lb in

Cylinder Head Bolt

•First Pass
30 N·m
22 lb ft

•Final Pass
155 degrees

Cylinder Head Front Chaincase Bolt
35 N·m
26 lb ft

Cylinder Head Oil Gallery Plug
35 N·m
26 lb ft

Drive Belt Tensioner Bolt
45 N·m
33 lb ft

EGR Cover Bolt
25 N·m
18 lb ft

Engine Coolant Temperature Sensor
20 N·m
15 lb ft

Engine Lift Bracket Front Bolt
25 N·m
18 lb ft

Engine Lift Bracket Rear Bolt
25 N·m
18 lb ft

EVAP Canister Valve Bolt
22 N·m
16 lb ft

Exhaust Manifold Pipe Flange Stud
16 N·m
12 lb ft

Exhaust Manifold to Cylinder Head Nut
14 N·m
124 lb in

Exhaust Manifold to Cylinder Head Stud
10 N·m
89 lb in

Flywheel Bolt - Automatic Transmission

•First Pass
53 N·m
39 lb ft

•Final Pass
25 degrees

Flywheel Bolt - Manual Transmission

•First Pass
53 N·m
39 lb ft

•Final Pass
25 degrees

Front Cover to Block Bolt
25 N·m
18 lb ft

Front Lift Bracket Bolt
25 N·m
18 lb ft

Fuel Pipe Bracket Bolt
10 N·m
89 lb in

Fuel Rail Bracket Stud
10 N·m
89 lb in

Generator to Block Bolt
23 N·m
17 lb ft

Heat Shield to Exhaust Manifold Bolt
22 N·m
16 lb ft

Ignition Coil Bolt
10 N·m
89 lb in

Intake Manifold to Cylinder Head Bolt
10 N·m
89 lb in

Intake Manifold to Cylinder Head Nut
10 N·m
89 lb in

Intake Manifold to Cylinder Head Stud
6 N·m
53 lb in

Knock Sensor Bolt
25 N·m
18 lb ft

Lower Crankcase to Block Perimeter Bolt
25 N·m
18 lb ft

Oil Cooler Bolts
22 N·m
16 lb ft

Oil Filter Housing Cover
22 N·m
16 lb ft

Oil Gallery Gerotor Cover - Rear Bolt
6 N·m
53 lb in

Oil Gallery Plug
35 N·m
26 lb ft

Oil Gallery Plug - Rear
60 N·m
44 lb ft

Oil Level Indicator Tube to Intake Manifold Bolt
10 N·m
89 lb in

Oil Pan Drain Plug
25 N·m
18 lb ft

Oil Pan to Block Bolts
25 N·m
18 lb ft

Oil Pressure Switch
22 N·m
16 lb ft

Oil Pump Cover Bolt
6 N·m
53 lb in

Oil Pump Pressure Relief Valve Plug
40 N·m
30 lb ft

Oxygen Sensor
42 N·m
31 lb ft

Piston Oil Squirter
15 N·m
11 lb ft

Power Steering Pump Bolt
25 N·m
18 lb ft

Spark Plug
20 N·m
15 lb ft

Starter Motor to Block Bolt
53 N·m
39 lb ft

Thermostat Housing to Block Bolts
10 N·m
89 lb in

Throttle Body Bolt
10 N·m
89 lb in

Throttle Body Nut
10 N·m
89 lb in

Throttle Body Stud
6 N·m
53 lb in

Timing Adjustable Chain Guide Bolt
10 N·m
89 lb in

Timing Chain Oil Nozzle Bolt
10 N·m
89 lb in

Timing Fixed Chain Guide Bolt
15 N·m
11 lb ft

Timing Upper Chain Guide Bolt
10 N·m
89 lb in

Vent Tube to Cylinder Head
15 N·m
11 lb ft

Water Jacket Drain Plug
20 N·m
15 lb ft

Water Pipe Support Bracket Bolt
10 N·m
89 lb in

Water Pump Access Cover Bolt
7 N·m
62 lb in

Water Pump/Balance Shaft Chain Tensioner Bolt
10 N·m
89 lb in

Water Pump Bolts
25 N·m
18 lb ft

Water Pump Sprocket Bolt
10 N·m
89 lb in

engine cooling system

Air Cleaner Assembly Nut (RPO LE5/L61)
10 N·m
89 lb in

Charged Air Cooler Bolt
10 N·m
89 lb in

Charged Air Cooler Pump Clamp Bolt
10 N·m
89 lb in

Charged Air Cooler Reservoir Bolt
8 N·m
70 lb in

Condenser Bolt
10 N·m
89 lb in

Coolant Heater Bolt
10 N·m
89 lb in

Coolant Heater Cord Clip Nut (RPO LSJ)
10 N·m
89 lb in

Engine Coolant Temprature (ECT) Sensor
20 N·m
15 lb ft

Exhaust Shield Stud
22 N·m
16 lb ft

Lower Radiator Mount Bracket Bolt
25 N·m
18 lb ft

Positive Crankcase Ventilation (PCV) Tube/Hose Bolt/Nut (RPO LSJ)
10 N·m
89 lb in

Surge Tank Bolt
6.5 N·m
58 lb in

Thermostat Cover Bolt
10 N·m
89 lb in

Thermostat Housing Bolt
10 N·m
89 lb in

Transmission Oil Cooler Line to Radiator
20 N·m
15 lb ft

Underhood Junction Block Bracket Bolt
25 N·m
18 lb ft

Underhood Junction Block Bracket Nut
10 N·m
89 lb in

Water Pump Access Plate Bolt
10 N·m
89 lb in

Water Pump Bolt
25 N·m
18 lb ft

Water Pump Sprocket Bolt
10 N·m
89 lb in

engine electrical
Battery Cable Ground Nut
20 N·m
15 lb ft

Battery Retainer Bolt
18 N·m
13 lb ft

Engine Harness Ground Nut
17 N·m
13 lb ft

Engine Harness Ground Stud
25 N·m
18 lb ft

Generator Bolt (RPO LSJ)
25 N·m
18 lb ft

Generator Bolt (RPOs L61/LE5)
22 N·m
16 lb ft

Generator Terminal Nut
20 N·m
15 lb ft

Junction Block Nut
17 N·m
13 lb ft

Negative Battery Cable Bolt
17 N·m
13 lb ft

Positive Battery Cable Bolt
17 N·m
13 lb ft

Starter Motor Bolt (RPO LSJ)
50 N·m
37 lb ft

Starter Motor Bolt (RPOs L61/LE5)
40 N·m
30 lb ft

Starter Solenoid Terminal Nut (RPO LSJ)
11 N·m
97 lb in

Starter Solenoid Terminal Nut (RPOs L61/LE5)
17 N·m
13 lb ft

Starter Solenoid S Terminal Nut
3 N·m
27 lb in

exhaust
Catalytic Converter-to-Exhaust Manifold Nuts
50 N·m
37 lb ft

Catalytic Converter-to-Muffler Pipe Nut
30 N·m
22 lb ft

Exhaust Clamp Nut
45 N·m
33 lb ft

Exhaust Manifold Heat Shield Stud
22 N·m
16 lb ft

Exhaust Manifold Nut
14 N·m
10 lb ft

Heater Oxygen Sensor (HO2S)
42 N·m
31 lb ft

Heat Shield Nut
10 N·m
89 lb in

Positive Crankcase Ventilation (PCV) Vent Tube Bolt/Nut (RPO LSJ)
10 N·m
89 lb in

Secondary Air Injection (AIR) Outlet Hose/Pipe Bracket Bolt (RPO L61)
20 N·m
15 lb ft

Secondary Air Injection (AIR) Outlet Hose/Pipe Bracket Nut (RPO L61)
10 N·m
89 lb in

Wheel Drive Shaft Heat Shield Bolt (to Engine Block)
30 N·m
22 lb ft

Wheel Drive Shaft Heat Shield Bolt (to Transaxle Mount Bracket)
10 N·m
89 lb in

---------- Post added at 12:55 PM ---------- Previous post was at 12:53 PM ----------

transmission getrag 5 speed f23
Drain Plugs
38 N·m
28 lb ft

Front Transmission Mount Bolts
50 N·m
37 lb ft

Front Transmission Mount Through Bolt
100 N·m
74 lb ft

Heat Shield to Engine Block Bolt
30 N·m
22 lb ft

Heat Shield to Transmission Mount Bolt
10 N·m
89 lb in

Intermediate Shaft Bolt
100 N·m
74 lb ft

Rear Cover Bolts
25 N·m
18 lb ft

Rear Transmission Mount Bracket to Transmission Bolts
60 N·m
44 lb ft

Rear Transmission Mount Through Bolt
100 N·m
74 lb ft

Rear Transmission Mount to Frame Bolts
60 N·m
44 lb ft

Reverse Lockout Bolt
6 N·m
53 lb in

Reverse Switch
18 N·m
13 lb ft

Ring Gear
90 N·m
66 lb ft

Side Transmission Mount Bolts
45 N·m
33 lb ft

Side Transmission Mount to Mid-Rail Bolts
34 N·m
25 lb ft

Shaft Bolts
100 N·m
74 lb ft

Shift Cable Attachment Nut
10 N·m
89 lb in

Shift Control Nuts
25 N·m
18 lb ft

Shifter Guide Bolt
25 N·m
18 lb ft

Shifter Retaining Bolts
25 N·m
18 lb ft

Speed Sensor
12 N·m
106 lb in

Transmission Housing Bolts
27 N·m
20 lb ft

UBEC Bracket Bolts
25 N·m
18 lb ft

UBEC Bracket Nuts
10 N·m
89 lb in

UBEC Positive Post
10 N·m
89 lb in

mu3 f35 5 speed manual
Backup Lamp Switch
24 N·m
18 lb ft

Ball Stud into Steering Knuckle
100 N·m
75 lb ft

Battery Cable Terminals
17 N·m
13 lb ft

Battery Feed Cable-to-Underhood Fuse Block
16 N·m
12 lb ft

Battery Hold Down Bracket Fasteners
20 N·m
15 lb ft

Battery Tray Fasteners
15 N·m
11 lb ft

Console Front Fasteners
2.5 N·m
22 lb in

Console Rear Fasteners
2.5 N·m
22 lb in

Control Assembly Rod-to-Control Lever Assembly Pinch Bolt

•First Pass
12 N·m
106 lb in

•Final Pass
180 degrees

Control Assembly-to-Floor Pan Bolts
8 N·m
71 lb in

Control Shaft Lever Pivot Pin Bracket-to-Transaxle Bolts
24 N·m
18 lb ft

Differential Bearing Retainer-to-Transaxle Bolts
24 N·m
18 lb ft

Exhaust Manifold Pipe-to-Exhaust Manifold
30 N·m
25 lb ft

Exhaust Manifold Pipe-to-Exhaust Manifold Nuts
50 N·m
37 lb ft

Exhaust Manifold-to-Resonator
20 N·m
15 lb ft

Fluid Level Check and Fill Plug
50 N·m
37 lb ft

Frame-to-Body Bolts

•First Pass
90 N·m
60 lb ft

•Second Pass
45 degrees

•Final Pass
60 degrees

Front Transaxle Mount Through Bolt
55 N·m
41 lb ft

Front Transaxle Mount-to-Transaxle
55 N·m
41 lb ft

Input Shaft Bearing Retainer Bolts
38 N·m
28 lb ft

Left Transaxle Mount Bolts
55 N·m
41 lb ft

Left Transaxle Mount Bracket-to-Transaxle Bolt
55 N·m
41 lb ft

Linkage Installer J 44015-to-Seat Tie Rod Ends
45 N·m
35 lb ft

Lower Control Arm Ball Stud Bolt and Nut-to-Steering Knuckle
100 N·m
75 lb ft

O2 Sensor Clamp-to-Heat Shield
4 N·m
35 lb in

Output Shaft Bearing Retainer Bolts-Inner
45 N·m
33 lb ft

Output Shaft Bearing Retainer Bolts-Outer
24 N·m
18 lb ft

Rear Cover-to-Transaxle Bolts
24 N·m
18 lb ft

Rear Transaxle Mount Through Bolt
90 N·m
66 lb ft

Rear Transaxle Mount-to-Frame Bolts
60 N·m
44 lb ft

Rear Transaxle Mount-to-Transaxle
55 N·m
41 lb ft

Reverse Idler Gear Shaft Bolt
24 N·m
18 lb ft

Reverse Idler Gear Shaft Bracket Bolt
24 N·m
18 lb ft

Reverse Shift Lever Bolts
24 N·m
18 lb ft

Right Front Lower Splash Shield to Frame Assembly
5 N·m
44 lb in

Slave Cylinder Fasteners
10 N·m
89 lb in

Stabilizer Bar Links-to-Strut Assemblies
65 N·m
50 lb ft

Steering Gear-to-Frame

•First Pass
45 N·m
35 lb ft

•Final Pass
90 degrees

Tie Rod End-to-Steering Knuckle
60 N·m
45 lb ft

Transaxle Case-to-Clutch Housing Bolts
24 N·m
18 lb ft

Transaxle Drain Plug
50 N·m
37 lb ft

Transaxle Fill Plug
50 N·m
37 lb ft

Transaxle Level Plug
50 N·m
37 lb ft

Transaxle Shift Control Shaft Block
24 N·m
18 lb ft

Transaxle-to-Engine Bolts
65 N·m
48 lb ft

Underhood Fuse Block Case Bolt
9 N·m
80 lb in

Wheel Bolts, Final Torque
125 N·m
92 lb ft

Wheel Bolts, Initial Torque
63 N·m
46 lb ft

Automatic Transaxle - 4T45-E

Bottom Pan to Case
81
12
M6x1.0x19.0
12 N·m
106 lb in

Channel Plate to Case
8
2
M6x1.0x63.0
14 N·m
124 lb in

Channel Plate to Case
25
6
M6x1.0x28.0
12 N·m
106 lb in

Channel Plate to Driven Sprocket Support
25
2
M6x1.0x28.0
14 N·m
124 lb in

Detent Spring Assembly to Channel Plate
805
1
M6x1.0x19.0
12 N·m
106 lb in

Drive Sprocket Support Assembly to Case
53
6
M6x1.0x17.0
12 N·m
106 lb in

Input Speed Sensor to Case
15
1
M6x1.0x15.0
12 N·m
106 lb in

Intermediate 4th Servo Cover to Case
25
3
M6x1.0x28.0
12 N·m
106 lb in

Line Pressure Plug to Case
48
1
1/8-27NPTF
12 N·m
106 lb in

Lo/Reverse Servo Cover to Case
25
3
M6x1.0x28.0
12 N·m
106 lb in

Oil Level Control Plug to Case
65
1
1/8-27NPTF
12 N·m
106 lb in

Oil Tube Assembly To Case
81
2
M6x1.0x19.0
12 N·m
106 lb in

Oil Tube Assembly to Forward Clutch
81
1
M6x1.0x19.0
12 N·m
106 lb in

Oil Tube Assembly to Lo/Reverse Servo Cover
81
1
M6x1.0x19.0
12 N·m
106 lb in

Output Shaft Speed Sensor to Case
61
1
M6x1.0x16.0
12 N·m
106 lb in

Pump to Case
9
1
M6x1.0x103.0
12 N·m
106 lb in

Pump to Channel Plate
8
1
M6x1.0x63.0
12 N·m
106 lb in

Pump to Channel Plate
12
6
M6x1.0x90.0
12 N·m
106 lb in

Side Cover to Case
3
10
M8x1.25x28.0
28 N·m
22 lb ft

Side Cover to Case (Stud)
4
1
M8x1.25x28.0
28 N·m
22 lb ft

Spacer to Driven Sprocket Support
20
2
M6x1.0x70.0
14 N·m
124 lb in

TFP Switch Assembly to Case
12
2
M6x1.0x90.0
14 N·m
124 lb in

TFP Switch to Channel Plate
16
3
M6x1.0x51.0
14 N·m
124 lb in

TFP Switch to Channel Plate
8
1
M6x1.0x63.0
14 N·m
124 lb in

Valve Body to Case
9
2
M6x1.0x103.0
14 N·m
124 lb in

Valve Body to Case
12
5
M6x1.0x90.0
14 N·m
124 lb in

Valve Body to Channel Plate
16
5
M6x1.0x51.0 
14 N·m
124 lb in

Wiring Harness Clip to Case
15
1
M6x1.0x15.0
12 N·m
106 lb in

---------- Post added at 12:58 PM ---------- Previous post was at 12:55 PM ----------

clutch
Clutch Actuator Bolts
10 N·m
89 lb in

Clutch Cover to Flywheel Bolts
30 N·m
22 lb ft

Clutch Pedal Bracket Nuts
10 N·m
89 lb in

Clutch Pedal Pivot Nut
10 N·m
89 lb in

Coolant Reservoir Attaching Nut
15 N·m
11 lb ft

seat belts
Child Tether Anchor Bolt
25 N·m
18 lb ft

Front Seat Belt Anchor Bolt
45 N·m
33 lb ft

Front Seat Belt Buckle Bolt
45 N·m
33 lb ft

Front Seat Retractor Bolt
45 N·m
33 lb ft

Rear Seat Belt Anchor Bolt
45 N·m
33 lb ft

Rear Seat Belt Buckle Nut
45 N·m
33 lb ft

Rear Seat Belt Retractor Bolt
45 N·m
33 lb ft

Shoulder Belt Height Adjuster Bolt
45 N·m
33 lb ft

SIR airbags
Forward Discriminating Sensor Bolt
8 N·m
71 lb in

Front Floor Console Armrest Bolt
10 N·m
89 lb in

Inflatable Restraint Roof Rail Module Bolt
8 N·m
71 lb in

Inflatable Restraint Roof Rail Module Screw
8 N·m
71 lb in

Inflatable Restraint Sensing and Diagnostic Module Nut
10 N·m
89 lb in

Instrument Panel Inflatable Restraint Module Bolt
10 N·m
89 lb in

Side Impact Sensor Bolt
8 N·m
71 lb in